The information listed below is a helpful guideline for commissioning and decommissioning your equipment.  These instructions are configured specifically for a Graco H25 machine, a Gap Pro gun equipped with a NAP AB Block Assembly, utilizing 2:1 divorced style drum pumps.  Although your setup may differ from this one, these operating instructions are a helpful guideline.

START UP

- Find 220 vac (2 – 110 Volt Circuit) Single Phase,  100 amps.

- Turn on breaker for lights in trailer.  Check oil level in compressor & pump lube        on Graco unit.  Change pump lube if discolored or fill if necessary.

- Turn breakers on for compressor & Graco pump.

- Turn on switch on side of Graco unit.

- Depress button to start hose heat on Graco unit.  This should take 15 – 20 minutes to come up to temperature.

- Flip Air Dryer rocker switch up in illuminated “ON” position.

- Depress red lever on air compressor pressure switch “DOWN” to initiate air compressor.

- Open main air outlet valve, located on the side of the air compressor.

- Check desiccant drierite cartridge in condition window, mounted to vent bung of drum.  (BLUE is ok, PINK is marginal, replace cartridge)

- Plug drum pump disconnects into each drum pump (2 total).  Listen & look for movement.

- Open bottom inlet valves on Graco machine.

- Prepare gun chamber outlet port by connecting air to gun.  Use appropriate size clean out drill, slowly insert into chamber & remove any debris until port is clean.  Air is always on when drilling front of the gun.

- Shut off air to gun.  Take gun & appropriate length of hose to the area where the application will be performed.

- Once hose has reached target temperature of 120 – 125 degrees, depress the “ON” button for “A” & “B” side preheater circuit.

- On right side of Graco pump, depress “ON” and the right button opposite of “PARK” to create hydraulic activation for spraying.  Check pressure gauges for final output of between 850 – 1100 PSI.  Gauges should be within 400 PSI of each other.

- Prepare with personal protection equipment & proceed to spray area with 5/16 nut driver in your pocket.

- Plug the air back into gun to activate the air purge.

- Open each side individually using the 5/16 nut driver.  Shut off valve and hold gun to dry surface to check for side seal leaking.  If  no “spotting” occurs, take a test spray and continue to perform job.  If “spotting” occurs, shut valves off & move to a clean area to remove side seal blocks & check for clean surface & make sure it’s free of scratches & debris.

NOTES

- Quick cycling ON/OFF on gun causes excessive heat on chamber & may deteriorate gun operation.

- Any changes in spray pattern/volume should prompt immediate termination of spraying & operator should inspect & clean front of chamber, very carefully.

- Do not rub build up from the front of the chamber barrel.  This action tends to caulk material IN.  Remove build up by using side flute of clean out drill, carefully rubbing it across the very front of chamber.

INTERMITTANT STOPPING OR BREAKS

- For pauses of 2 hours or less the following steps should be practiced.

- Shut 5/16 valves on gun, allow to purge air for 3 – 5 minutes.  Disconnect air & inject encapsulation grease into tip of gun.

- Turn hydraulic action on machine to “OFF”.  Turn off preheater “A” & “B”.

- Close pump inlet valves.

- Hose may remain “ON”.

- Air compressor may be ON or OFF.

END OF DAY

- Place hydraulic circuit on Graco in “PARK” position by depressing button on left side of action switching area.

- Spray gun until pattern reduces, DO NOT exhaust pressure completely.  Machine hydraulics circuit shall turn off when retracted to “PARK” (far left position)  also engage safety on gun.

- Shut off all 5/16 valves on gun & AB Block Assembly.  Depress & turn gun safety lever to “ON” position.

- Turn of preheaters, heated hose, main ON/OFF switch located on bottom right of machine.  Shut both inlet valves for “A” & “B” supply from pumps.  Remove quick coupling disconnects which supply air to the drum pumps.

- Wind heated hose assembly up while air is still purging from the gun.  Place gun handle in vise and disconnect air.  Clean front barrel of chamber.  Carefully, insert grease.  Hold rag over end of gun barrel & reconnect air.  Check that the gun safety is in “ON” position and trigger gun once, while in the safety position.

- Disconnect air to gun and reapply grease in front barrel of gun.

- Shut off air compressor, electric and main valve on side of compressor storage tank.

- Turn compressed air dryer to “OFF” by moving illuminated rocker switch down until un-illuminated.

- Check all stowage items to make sure they are secured.

- Place resin drum pump in caddy if applicable.  Return resin bung caps on both 2” & ¾”.  Tighten with bung wrench to retain blowing agent.

Copyright © 2006 North American Processing